TL1 – TL2 Layer Adaptation

V1.0.1 – July 18, 2003

Yann Bajot, Prosilog

Stéphane Guntz, Prosilog

OCP International Partnership (OCP-IP) disclaims all warranties and liability for the use of this document and the information contained herein and assumes no responsibility for any errors that may appear in this document, nor does OCP-IP make a commitment to update the information contained herein.

Contact the OCP-IP office to obtain the latest revision of this document.

Questions regarding this document or membership in OCP-IP may be forwarded to:

OCP-IP

www.ocpip.org

E-mail: admin@ocpip.org

Phone: +1 503-291-2560

Fax: +1 503-297-1090

OCP-IP Technical Support

techsupport@ocpip.org

DISCLAIMER

This OCP-IP document is provided "as is" with no warranties whatsoever, including any warranty of merchantability, noninfringement, fitness for any particular purpose, or any warranty otherwise arising out of any proposal, specification or sample.  OCP-IP disclaims all liability for infringement of proprietary rights, relating to use of information in this document.  No license, express or implied, by estoppel or otherwise, to any intellectual property rights is granted herein.

All product names are trademarks, registered trademarks, or servicemarks of their respective owners.

Copyright (C) 2003 OCP-IP

# Introduction

Layer adapters presented here are intended to illustrate the TL1-TL2 layer adaptation process for the OCP-based transaction level communication interface.

These adapters are given as examples and implement only a subset of the whole OCP 1.0 dataflow interface. Note that we only consider the dataflow part of the OCP interface here (no sideband signals).

# TL1 MASTEr Adapter

This adapter enables the connection of a TL1 master with a TL2 slave, through a TL1 channel on one side, and a TL2 channel on the other side.

Figure 1: TL1 Master Adapter

One example shows the use of the TL1 master adapter for different kinds of request.

The adapter’s code can be found in the following files: ocp_tl1_tl2_master_adapter.cpp and ocp_tl1_tl2_master_adapter.h.

## Template parameters

The TL1 Master Adapter is templatized over 2 parameters:

- the TL1 data class

- the TL2 data class

The two data classes must be the same as the ones used respectively by the TL1 master and channel, and the TL2 slave and channel. It can be the default data classes:

- TL1_TEMPL_DATA_CL for the TL1 data class,

- OCP_TL2_TEMPL_DATA_CL for the TL2 data class.

## Constructor

The adapter’s constructor takes 3 parameters:

OCP_TL1_TL2_Master_Adapter(sc_module_name name

, int max_burst_length

, int adapter_depth

)

- name : name of the module instance

- max_burst_length: maximum size of a burst

- adapter_depth: number of TL2 requests/TL2 responses that the adapter stores internally. It indicates how many successive request/response can be stored by the adapter, while processing the current request/response. When the adapter has stored adapter_depth requests or responses that need to be processed, it waits for the end of the current request/response before releasing the request/response channel.

## Interface

The TL1 master adapter has 3 ports:

- a TL1 slave port

  - name : SlaveP

  - type : sc_port<TLslaveIF<TdataCl_tl1>, 1> (TdataCl_tl1 is the template parameter of the module corresponding to the TL1 data class)

- a TL2 master port

  - name : MasterP

  - type : sc_port<TLmasterIF<TdataCl_tl2>, 1> (TdataCl_tl2 is the template parameter of the module corresponding to the TL2 data class)

- a clock port (for the TL1 side of the adapter, it is the same clock as the one used by the TL1 master)

  - name : clk

  - type : sc_in_clk

The TL1 slave port must be connected to the TL1 channel, the TL2 master port to the TL2 channel, and the clock port to the corresponding clock signal.

## Functionality

The adapter is totally pipelined, i.e. the request and response threads are totally independent.

Note: only incrementing and streaming bursts can be handled by the adapter. Custom bursts will be separated and sent as single TL2 requests to the slave (see OCP 1.0 specification for more details about the type of bursts).

Basically, the adapter’s purpose is to get some TL1 requests, pack them in a burst (if needed), send a corresponding TL2 request to the TL2 slave. On the response side, it gets some TL2 responses, separates them in successive TL1 responses that are sent to the TL1 master.

To print debug messages on the screen, uncomment the following line in the opc_tl1_tl2_master_adapter.h file:

#define DEBUG_MASTER_ADAPTER_TL1

More precisely, during an OCP transaction, the adapter behaves as follows:

For the request thread:

The TL1 master:

- sends some TL1 requests, which are part (or not, in the case of a single request) of a burst. These requests have certain characteristics, defined by the request fields (MAddr, MBurst, MAddrSpace, MByteEn, MCmd, MConnID, MThreadID). For an incrementing request burst, these characteristics (except for the MAddr and MBurst fields) always stay the same. The last command of the burst is specified by the OCP_MBURST_LAST value for the MBurst field.

The TL1 Master Adapter:

- catches the TL1 request, stores its characteristics (MAddr, MAddrSpace, MBurst, MCmd…+data field),

- compares these characteristics with the characteristics of the burst currently stored, if any burst was being stored. If not, the adapter just stores the new request parameters.

The adapter can behave in four different ways, depending on the request fields, and if the current request is the beginning of a new burst. It can:

- send the current burst WITH the last request,

- or send the current burst WITHOUT the last request,

- or store the last request (which becomes a part of the current burst),

- or send the current burst without the last request and a single request.

If the new request is the beginning of a new burst (or a single request): the adapter sends the request to the slave (if it is a single request), or stores it, depending on the MBurst field.

If a current burst is being stored in the adapter:

- the adapter sends the current burst with the new command (if the request fields have not changed, the increment of address is right (see below for more details), and the MBurst field is OCP_MBURST_LAST, or if the completed burst has reached maximum length),

- the adapter sends the current burst without the new command (if the request fields have changed, or the increment of address does not correspond to the current burst increment, or the MBurst field indicates another type of burst (for example, change from a STRM value to a CONT value) and is not OCP_MBURST_LAST),

- the adapter stores the new command internally, which becomes a part of the current burst,

- the adapter sends the current burst without the last request, and sends the last request as a single request (if the request fields have changed and the MBurst field of the last request is OCP_MBURST_LAST)

The increment of address should be:

- for a incrementing burst: the increment corresponding to the OCP data width,

- for a streaming burst: 0.

When the adapter has to send a TL2 request to the slave, corresponding to the TL1 requests that it has received, the adapter:

- specifies the TL2 request parameters,

- increments the number of pending TL2 requests that need to be sent to the slave,

- releases the TL1 channel, if the request buffer is not full, so that the master can send other TL1 requests while the TL2 request is being processed. If the request buffer is full, the adapter has to wait for a TL2 request to be processed by the slave, in order to store new requests coming from the TL1 master,

- makes some blocking calls on the TL2 master port (MputWriteRequestBlocking() or MputReadRequestBlocking()),

- decrements the number of pending TL2 requests, when the blocking call returns (meaning that the slave has accepted the request),

- if it was not done before, releases the TL1 channel when the blocking call returns.

If there is a pending TL2 request (or several pending requests) that needs to be sent to the slave, the adapter makes other blocking put*RequestBlocking() calls corresponding to these requests, as soon as the current TL2 request has been accepted by the slave.

For the response thread:

The TL1 Master Adapter issues a blocking MgetResponseBlocking() call, and waits for a new TL2 response sent by the slave.

When a new TL2 response arrives, the TL1 master adapter:

- transfers the response fields from the TL2 data class to the adapter,

- copies the response data fields internally,

- increments the number of groups of TL1 responses that need to be sent to the master (one TL2 response corresponds to one group of TL1 responses),

- releases the TL2 channel, if the response buffer is not full, so that the slave can send other TL2 responses while the group of TL1 response is being processed. If the response buffer is full, the adapter has to wait that one group of TL1 response has been processed by the master, in order to store new responses coming from the TL2 slave.

Then the master adapter issues the corresponding TL1 responses to the TL1 master. It:

- copies the response fields into the TL1 data class

- sends the group of TL1 responses at each edge of the clock

- releases the response thread when all the responses have been sent, if it was not done before,

- decrements the number of group of TL1 responses that needs to be sent to the master.

If there are some pending groups of TL1 responses that need to be sent to the master, the master adapter send them when the current group of TL1 request has been sent. Otherwise, the adapter waits for another group of TL1 responses to send.

## Example

The example demonstrates the connection of a TL1 master to a TL2 slave, through a TL1 master adapter and two channels (TL1 and TL2 channel).

The master and slave can be found in the following files:

- ocp_tl1_master.cpp and ocp_tl1_master.h,

- ocp_tl2_slave.cpp and ocp_tl2_slave.h,

The top-level can be found in the top_tl1_tl2_master_adapter.cpp file. Use the Makefile.master_adapter to build the example, and run run_master_adapter executable.

The master adapter is templatized over the two default data classes TL1_TEMPL_DATA_CL and OCP_TL2_TEMPL_DATA_CL. The parameters of the adapter are the following:

- name of the instance: "master_adapter"

- maximum burst length: 8

- Adapter depth: 6

The TL1 master successively issues the following requests, at each rising edge of the clock:

- A write burst composed of 11 atomic requests (data written are 0 to 10). The address increment is usually 4 because the data is 32-bit wide, except for the fourth request, where the address increment is 5.

- A read burst composed of 5 atomic requests (the address increment is always 4)

- One single read request

- One single write request (data written =17)

The adapter separates the first write burst in two bursts:

- The first burst has a length of 3 (the firsts three write requests with the same address change)

- The second burst has a length of 8 (maximum burst length)

It then sends the read burst  (length=5) and the corresponding read response from the slave to the master. The adapter converts the TL2 response in 5 atomic TL1 requests (read data are 11 to 15).

At the same time, the master issues its single read request. The TL1 request is converted into a TL2 read request. The slave issues its response, which is stored internally until the burst read is finished. When the 5th and last read request is accepted by the master, the adapter can send the response corresponding to the single read request (datum read=16).

The master then finishes with its last single write request.

# TL1 SLAVE Adapter

This adapter enables the connection of a TL2 master with a TL1 slave, through a TL2 channel on one side, and a TL1 channel on the other side.

Figure 2: TL1 Slave Adapter

One example shows the use of the TL1 master adapter for different kinds of request.

The adapter’s code can be found in the following files: ocp_tl1_tl2_slave_adapter.cpp and ocp_tl1_tl2_slave_adapter.h.

## Template parameters

The TL1 Slave Adapter is templatized over 2 parameters:

- the TL1 data class

- the TL2 data class

The two data classes must be the same as the ones used respectively by the TL1 master and channel, and the TL2 slave and channel. It can be the default data classes:

- TL1_TEMPL_DATA_CL for the TL1 data class,

- OCP_TL2_TEMPL_DATA_CL for the TL2 data class.

## Constructor

The adapter’s constructor takes 4 parameters:

OCP_TL1_TL2_Master_Adapter(sc_module_name name

, int max_burst_length

, int adapter_depth

, bool writeResponse)

- name : name of the module instance

- max_burst_length: maximum size of a burst

- adapter_depth: number of TL2 requests/TL2 responses that the adapter stores internally. It indicates how many successive request/response can be stored by the adapter, while processing the current request/response. When the adapter has stored adapter_depth requests or responses that need to be processed, it waits for the end of the current request/response before releasing the request/response channel.

- writeResponse: indicates if the slave connected to the adapter sends responses to write requests. In that case, the adapter must send these responses to the master.

## Interface

The TL1 slave adapter has 3 ports:

- a TL2 slave port

  - name : SlaveP

  - type : sc_port<TLslaveIF<TdataCl_tl2>, 1> (TdataCl_tl2 is the template parameter of the module corresponding to the TL2 data class)

- a TL1 master port

  - name : MasterP

  - type : sc_port<TLmasterIF<TdataCl_tl1>, 1> (TdataCl_tl1 is the template parameter of the module corresponding to the TL1 data class)

- a clock port (for the TL1 side of the adapter, it is the same clock as the one used by the TL1 master)

  - name : clk

  - type : sc_in_clk

The TL2 slave port must be connected to the TL2 channel, the TL1 master port to the TL1 channel, and the clock port to the corresponding clock signal.

## Functionality

The adapter is totally pipelined, i.e. the request and response threads are totally independent.

The slave adapter and the master adapter have a symmetrical behaviour: the response threads in one adapter correspond to the request threads in the other adapter. What is different is that the slave adapter has to know how many TL1 responses correspond to the same TL2 burst, to know how to pack them (it is similar to the MBurst field on the request side for the master adapter). We assume that the TL1 responses are safely handled to the adapter, and that no responses are lost.

Basically, the slave adapter’s purpose is to get some TL2 request, separates it in corresponding TL1 requests and sends them to the TL1 slave. On the response side, it gets some TL1 responses, packs them in burst (if needed) and sends it to the TL2 master.

To print debug messages on the screen, uncomment the following line in the opc_tl1_tl2_slave_adapter.h file:

#define DEBUG_SLAVE_ADAPTER_TL1

More precisely, during an OCP transaction, the adapter behaves as follows:

For the request thread:

The TL1 Slave Adapter issues a blocking SgetRequestBlocking() call, and waits for a new TL2 request sent by the master.

When a new TL2 request arrives, the TL1 slave adapter:

- transfers the request fields from the TL2 data class to the adapter,

- copies the request data fields internally,

- checks if the size of the burst is not greater than the maximum burst length. On the contrary, it issues a message and truncates the end of the request burst.

- increments the number of groups of TL1 requests that need to be sent to the master (one TL2 request corresponds to one group of TL1 requests),

- releases the TL2 channel, if the request buffer is not full, so that the master can send other TL2 requests while the group of TL1 request is being processed. If the request buffer is full, the adapter has to wait that one group of TL1 request has been processed by the master, in order to store new requests coming from the TL2 master,

Then the slave adapter issues the corresponding TL1 requests to the TL1 slave. It:

- copies the requests fields into the TL1 data class,

- sends the group of TL1 requests at each edge of the clock,

- releases the request thread when all the requests have been sent, if it was not done before,

- decrements the number of group of TL1 requests that need to be sent to the slave.

If there are some pending groups of TL1 requests that need to be sent to the slave, the master adapter send them when the current group of TL1 request has been sent. Otherwise, the adapter waits for another group of TL1 responses to send.

For the response thread:

The TL1 slave:

- sends some TL1 responses, which are part (or not, in the case of a single response) of a burst. These responses have certain characteristics, defined by the response fields (SResp, SThreadID). The number of responses which compose the burst is known by the adapter (from the TL2 request it has sent before)

The TL1 Slave Adapter:

- catches the TL1 response, stores its characteristics (SResp, SThreadID +data field),

- does the same with the other responses until the last response of the burst has arrived.

The adapter can behave in three different ways, depending on the response fields, and if the current request is the beginning of a new burst. It can:

- send the current burst WITH the last response,

- or send the current burst WITHOUT the last response,

- or store the last response (which becomes a part of the current burst).

If a current burst is being stored in the adapter:

- the adapter sends the current burst with the new command (if the response fields have not changed, and the new response is the last of the burst),

- the adapter sends the current burst without the new command (if the response fields have changed,

- the adapter stores the new command internally, which becomes a part of the current burst.

When the adapter has to send a TL2 response to the slave, corresponding to the group of TL1 responses that it has received, the adapter:

- specifies the TL2 response parameters,

- increments the number of pending TL2 responses that need to be sent to the slave,

- releases the TL1 channel, if the response buffer is not full, so that the slave can send other TL1 responses while the TL2 response is being processed. If the response buffer is full, the adapter has to wait for a TL2 response to be processed by the master, in order to store new responses coming from the TL1 slave,

- makes some blocking calls on the TL2 slave port (SputResponseBlocking()),

- decrements the number of pending TL2 responses, when the blocking call returns (meaning that the master has accepted the response),

- if it was not done before, releases the TL1 channel when the blocking call returns.

If there is a pending TL2 response (or several pending responses) that needs to be sent to the master, the adapter makes other blocking SputResponseBlocking() calls corresponding to these responses, as soon as the current TL2 response has been accepted by the master.

## Example

The example demonstrates the connection of a TL2 master to a TL1 slave, through a TL1 slave adapter and two channels (TL1 and TL2 channel).

The master and slave can be found in the following files:

- ocp_tl2_master.cpp and ocp_tl2_master.h,

- ocp_tl1_slave_sync.cpp and ocp_tl1_slave_sync.h,

The top-level can be found in the top_tl1_tl2_slave_adapter.cpp file. Use the Makefile.slave_adapter to build the example, and run run_slave_adapter executable.

The slave adapter is templatized over the two default data classes TL1_TEMPL_DATA_CL and OCP_TL2_TEMPL_DATA_CL. The parameters of the adapter are the following:

- name of the instance: "slave_adapter"

- maximum burst length: 16

- Adapter depth: 3

- WriteResponse: false (the slave does not send a response to a write request)

The TL2 master successively issues the following requests:

- A write burst composed of 10 atomic requests (data written are 1, 3, 5…). The address increment is usually 4 because the data is 32-bit wide. The base address is 0.

- 2 nanoseconds after the first burst has been accepted, the master sends a read burst composed of 10 atomic requests (the address increment is always 4 and the base address is 0). The read data are those which have been written by the previous burst.

The adapter separates the first write burst in 10 atomic TL1 requests, and the read burst in 10 atomic read requests. It then retrieves the 10 responses that the slave has sent, and eventually sends the corresponding TL2 response to the master.

To see the effect of the adapter_depth parameter: you can change it to 1, meaning that the slave adapter can only store one TL2 request/response at a time. The adapter releases the request channel when it has finished sending the 10 TL1 requests to the slave. When adapter_depth is equal or superior to 2, the request channel is released immediately after the first TL2 request has been stored by the adapter.